AprilTags and Accessors API in Ptolemy II

Implementing in Ptolemy II (CapeCode)

To implement an automated visual tracking loop with AprilTags and the Accessors API in Ptolemy II, you will use the CapeCode configuration. CapeCode is the specialized host environment inside Ptolemy II designed explicitly for composing JavaScript-based IoT accessors using visual dataflow semantics.

The Block Diagram Architecture

[ Clock ] ---> [ Camera (Accessor) ] ---> [ AprilTags (Accessor) ] ---> [ TextDisplay / Actuator ]

Step-by-Step Implementation in Vergil

Step 1: Set up the Director

Open Vergil and create a new model layout.

From the left palette, navigate to Directors and drag a DE Director (Discrete Event) or SDF Director into your workspace.

(Optional) If using DE, add a Clock actor to trigger execution at a stable interval.

Step 2: Add the Camera Accessor

Navigate through the library tree: CapeCode -> Accessors -> cameras -> Camera.

Drag the Camera accessor onto the canvas.

Double-click the component and modify parameters: Set maxFrameRate to 5 (5 frames per second keeps latency minimal).

Step 3: Add the AprilTags Accessor

Go to CapeCode -> Accessors -> image -> AprilTags.

Drag the AprilTags accessor onto the canvas.

Connect the image output port of your Camera accessor directly to the image input port of the AprilTags accessor.

Step 4: Capture and Route the Outputs

output: Emits the modified image overlay showing colored borders highlighting any detected tags.

tags: Emits an Array of JSON Objects containing structured coordinate data for every detected tag.

5. Understanding the Structured Data Layout

When an AprilTag is detected, the array sent to the tags output port structural properties look like this:

[
  {
    "id": 19,
    "center": [240.5, 320.2],
    "perimeter": [,,,]
  }
]

6. Injecting Custom Automation Control Logic

Place a JavaScript actor (Actors -> Scripting -> JavaScript) directly after the AprilTags output to compute motor orientation vectors:

exports.setup = function() {
    this.input('tags');
    this.output('steeringError');
};

exports.initialize = function() {
    this.addInputHandler('tags', function() {
        var detectedTags = this.get('tags');
        if (detectedTags && detectedTags.length > 0) {
            var targetTag = detectedTags[0]; 
            var tagX = targetTag.center[0];
            var frameCenter = 320; 
            var error = tagX - frameCenter; 
            this.send('steeringError', error);
        } else {
            this.send('steeringError', 0);
        }
    });
};

Note: A complete reference demo configuration is available inside Ptolemy II at: $PTII/ptolemy/demo/CapeCode/CapeCode1.xml